# ODrive S1 Bench Validation Procedure — Keya Motor

Purpose: Confirm that one ODrive S1 correctly drives one Keya gearmotor (phase output + Hall feedback) before it replaces the Keya driver in the traction system.

Scope: Validate one motor/driver pair at a time. Repeat the whole procedure for the second unit — do not wire both up simultaneously on your first pass.

## 0. Before You Start — Safety

- No load on the shaft. If the motor is mounted on the robot, lift the wheel off the ground or fully disconnect it from the drivetrain. The first spin must be free-running.

- Power supply off while wiring. Only apply DC power once every connection below is checked.

- Verify polarity before energizing. DC+/DC- reversed will damage the board — the datasheet calls this out explicitly.

- Have a way to cut power fast (supply switch or accessible plug) within arm's reach during the first spin.

- Stay clear of the shaft and coupling once power is applied, even at low commanded speed.

- USB and DC power together: only if you have a USB isolator. Without one, don't have the USB cable and the main DC supply connected at the same time to avoid a ground loop.

## 1. Physical Connections

Use the pinout images from CONNECTIONS_DRIVER_ODrive_S1.pdf as the source of truth for exact terminal/pin locations on your boards. This section tells you which Keya wire goes to which ODrive signal.

### 1.1 Motor phases (power terminal block)

| Keya wire | Keya function | ODrive S1 terminal |
| --- | --- | --- |
| Red | Phase U | A |
| Blue | Phase V | B |
| Black | Phase W | C |

Also connect:

- DC+ and DC- to your bench power supply (12–50.5 V, verify polarity before switching on).

- R+ and R- to the included 2Ω 50W brake resistor. Leave this connected even for a no-load bench test — it's cheap insurance and you'll need it later regardless.

Per ODrive's documentation, phase order A/B/C doesn't have to match any particular convention — the motor will still spin, just possibly in the opposite direction from what you expect. If it turns the "wrong" way during the first spin, don't stop to rewire — see Step 4.

### 1.2 Hall sensor feedback (J11 connector)

| Keya wire | Keya function | ODrive S1 J11 signal |
| --- | --- | --- |
| Red | +5V | 5V |
| Yellow | Hall U | HALL A |
| Green | Hall V | HALL B |
| Blue | Hall W | HALL C |
| Black | 0V | GND |

Locate these five signals on your J11 pinout diagram by their printed labels (5V, HALL A, HALL B, HALL C, GND) rather than by pin number — cross-check against the ODrive S1 Datasheet pinout table if anything on the sticker/silkscreen is ambiguous, since pin index labeling has shifted slightly across hardware/documentation revisions.

### 1.3 Connect USB

Plug the ODrive S1 into your computer via USB-C. This is how you'll configure and calibrate it — you don't need CAN wired up for this validation pass.

## 2. Software Setup

You have two options; either is fine for this procedure.

- Web GUI (recommended for this first pass): go to the ODrive Web GUI in a WebUSB-compatible browser (Chrome or Edge — not Firefox). No install required. It walks you through configuration and calibration with a wizard.

- odrivetool: the Python command-line tool, better if you want to script the configuration or inspect live values directly. Install per the odrivetool setup guide.

Don't run both at once — the ODrive can only be claimed by one USB client at a time.

For this first validation, use the Web GUI wizard for configuration and calibration, and keep odrivetool in your back pocket for Step 5 if you want direct numeric feedback (bus voltage, velocity, currents) during the spin test.

## 3. Driver Configuration

Power on the DC supply, then open the Web GUI. It should detect the board and start the setup wizard. Configure:

- Motor type: the wizard offers a small set of motor type presets (e.g. high-current vs. gimbal-style). Pick based on the Keya motor's actual electrical characteristics — check its datasheet against the wizard's descriptions rather than assuming which one applies.

- Pole pairs: check the Keya motor's datasheet or nameplate for this value — it's motor-specific and the wizard can't guess it. If it's not documented anywhere, this is worth flagging back to whoever specced the motor before you go further, since a wrong pole pair count will make calibration and every current/torque number after it wrong.

- Feedback type: select Hall Effect Sensors (not incremental encoder).

- Current limit: set conservatively for this first test — well under the motor's rated continuous current — you can raise it later once the driver is validated.

- DC bus voltage limits: confirm they match your bench supply.

Save this configuration before moving to calibration.

## 4. Motor Calibration

Run the calibration sequence from the wizard (or manually via odrivetool if you're using that path). It runs in this order:

- Motor calibration — measures phase resistance and inductance. The motor will click/hum briefly; it should not spin.

- Hall polarity / encoder offset calibration — this one does rotate the shaft a bit to find the offset between the electrical phase and the Hall signal. This is why the "no load, shaft free" rule in Step 0 matters — gravity or spring loads during this step can throw the calibration off or cause a jerky motion.

After calibration completes, check for errors in the GUI (or odrv0.axis0.errors in odrivetool). Zero errors means calibration succeeded. Save this to persistent memory so it survives a reboot.

If the motor turns the wrong direction during Step 5 below, don't rewire the phase wires — instead flip the direction convention in software (documented in the ODrive Hardware Configuration guide) and recalibrate.

## 5. First Spin — No Load

- Set the axis to closed-loop control (via the GUI or AxisState.CLOSED_LOOP_CONTROL in odrivetool).

- Command a small velocity — start low, something like a slow, easily-observed rotation rather than anything near the motor's rated speed.

- Watch and listen: rotation should be smooth and quiet, not cogging, buzzing, or stuttering.

- If using odrivetool alongside the GUI test, watch vbus_voltage, motor.current_control.Iq_measured, and encoder.vel_estimate (or the Hall equivalent) live — this gives you a numeric record, not just a visual check.

- Try commanding the opposite direction and confirm it responds correctly both ways.

- Stop the motor (return to idle) between direction changes rather than reversing at speed.

## 6. Validation Checklist

Declare the driver validated for this motor only once all of these are true:

- Motor calibration and Hall offset calibration both completed with zero errors

- Motor spins smoothly in both directions at low commanded velocity, no load

- No unexpected noise, cogging, or stalling

- Current draw at no load is low and stable — if it's not, stop and re-check the phase and Hall wiring against Section 1 before continuing

- No fault flags appear during a few minutes of running (thermal, overcurrent, encoder/Hall error)

- Calibration saved to persistent memory and survives a power cycle (reboot the ODrive, confirm it comes back up correctly without recalibrating)

Once both units pass, they're ready for integration testing under actual load.

References

- ODrive Getting Started guide — https://docs.odriverobotics.com/

- ODrive S1 Datasheet (pinout table) — https://docs.odriverobotics.com/v/latest/hardware/s1-datasheet.html

- odrivetool Setup guide — https://docs.odriverobotics.com/v/latest/guides/odrivetool-setup.html

- ODrive Hardware Configuration guide (direction, thermal limits) — https://docs.odriverobotics.com/v/latest/manual/hardware-config.html

- ODrive Web GUI — https://gui.odriverobotics.com