Assumptions & Design Decisions

Sunnybotics Robot Health Simulator

The Robot

I modeled the robot with the following assumed specs based on information from what was provided during the technical assessment.

| Parameter | Value |
| --- | --- |
| Supply voltage | 24V |
| Battery output current | 30A max |
| Battery Type | Lithium |
| Autonomy per cycle | 4.5h |
| Total operational weight | 50 kg |
| Cleaning efficiency | 875 m²/h |
| Pump pressure | 3 bar |
| Water consumption | 0.5 L/m² |
| Water system | 10 active sprinklers |
| Traction system | Vulcanized rubber tracks |
| Operation mode | Teleoperated / Autonomous |
| Starting battery level | 70–95% |
| Starting tank level | 15–20L |
| Starting motor temp | 28–35°C |

Operating States

CHARGING → NAVIGATING → CLEANING → NAVIGATING → CLEANING → NAVIGATING → CLEANING → RETURNING → CHARGING

Each state has different expected sensor behavior in terms of current draw, brush activity, pump pressure, and battery drain. The simulation samples sensor values every 5 seconds, which balances data resolution against dataset size.

Simulated Sensors

| Sensor | Normal Range | Basis |
| --- | --- | --- |
| Battery % | 70–100% | Drains during operation, charges at base |
| Brush RPM | 0 or ~1000 RPM | Only active during cleaning |
| Pump pressure | 0 or ~3.0 bar | Based on real spec (3 bar system) |
| Water tank level | 0–20L | Drains at 0.5 L/m² per spec |
| Motor temperature | 28–80°C | Rises during cleaning, cools at rest |
| Current draw | 2–30A | Based on 24V / 30A |

### Current Draw by State

Based on the real 24V / 30A spec, current draw was estimated per operating state:

| State | Current | Power | Reasoning |
| --- | --- | --- | --- |
| Charging | 2A | 48W | Only electronics ticking over |
| Navigating | 8A | 192W | Rubber tracks running, no brush or pump |
| Cleaning | 18A | 432W | Tracks + brush motor + 10 sprinkler pump |
| Returning | 7A | 168W | Lighter load, tank partially drained |

### Fault Detection Rules

I chose to detect 5 faults to begin with

| Fault | Trigger Condition | Reasoning |
| --- | --- | --- |
| Brush Stall | RPM < 200 AND current > 20A during cleaning | Motor fighting blockage — RPM drops, current spikes |
| Water Tank Empty | Tank level < 1L during cleaning | 1L buffer accounts for sensor noise |
| Motor Overheat | Motor temperature > 80°C | 5°C buffer above normal max of 75°C |
| Low Pump Pressure | Pressure < 1.0 bar during cleaning AND tank not empty | Distinguishes clogged nozzle from empty tank |
| Low Battery | Battery < 20% while not charging | Gives operator time to return to base |

### Fault Injection

Each simulation run randomly decides whether each fault occurs and where in the session it starts. The faults are injected by modifying sensor values from a random point forward and continue to degrade gradually rather than instantly to create a more realistic simulation. Multiple faults can occur in a single run, and some runs can also be completely clean.

### How This Applies to Real Operations

The architecture of this simulator maps directly to a real deployment. The sensor generator would be replaced by live telemetry from the robot over MQTT or a REST API, the fault detection rules would run on incoming data in real time, and the dashboard would connect to a live stream instead of a simulated dataset.

One thing that stood out to me while building this is how much can go wrong in a single session that an operator would not be able to catch without data. Things like a brush stalling mid-row, a leaking water tank, or a motor overheating are not always immediately obvious from the outside. By the time someone notices, the damage is already done.

What a system like this basic simulator does is give operators eyes on the robot at all times. The moment something drifts outside of normal range, an alert is fired. That means the difference between catching a brush stall early and catching it after the motor burns out. At scale, across a fleet of robots operating across a large solar portfolio, that visibility is imperative. No team can manually watch every robot, so the monitoring layer can take over for them.

Long term, every session this system runs generates data about current draw patterns, temperature curves, and how fast the tank drains across different conditions. Over time that data can help Sunnybotics detect issue patterns like a pattern in how motors that run hotter than average in the first hour tend to stall before the session ends or battery drainage accelerates in certain ambient temperatures. These patterns would not be visible to the human eye, but can be something an ML model is trained to detect and diagnose.